Hand Cream

Formulation reference: FM00050/A

| Phase | Trade Name | INCI Ingredients | Function | % w/w | Supplier |
| --- | --- | --- | --- | --- | --- |
| A | Deionised Water | Aqua | Solvent | Up to 100% |  |
|  | Edeta BX Powder | Tetrasodium EDTA | Chelating Agent | 0.05 | BASF |
| B | Keltrol RD | Xanthan Gum | Thickener | 0.10 | Azelis |
| C | Pricerine 9091 | Glycerin | Humectant | 5.00 | Croda |
| D | Agenaflo OS 9051 | Aluminium Starch Octenylsuccinate | Softening/ Mattifying | 1.00 | Azelis |
| E | Lanette O | Cetearyl Alcohol | Wax | 2.00 | BASF |
|  | Q-SOFT-GMS-SE | Glyceryl Stearate SE | Emulsifier | 2.00 | OQEMA |
|  | Olivem 1000 | Cetearyl Olivate (and) Sorbitan Olivate | Emulsifier | 2.00 | Hallstar |
|  | Cetiol SB 45 | Butyrospermum Parkii (Shea) Butter | Butter/ Moisturising | 3.00 | BASF |
|  | Seatons Cocoa Butter | Theobroma Cocao Butter | Butter/ Moisturising | 2.00 | Croda |
|  | Cupuacu Butter Ultra Refined | Theobroma Grandiflorum Seed Butter | Butter/ Moisturising | 1.00 | Hallstar |
|  | Pionier 2076P | Light Liquid Paraffin | Moisturising | 3.00 | OQEMA |
|  | Isopropyl Myristat | Isopropyl Myristate | Emollient | 4.00 | BASF |

Leave hands feeling soft and smooth with this rich moisturising hand cream. Containing Vitamin B5 to help protect and strengthen nails. Formulated with natural-based Texique HE10 to provide thickening and viscosity stability with a unique textural benefit. Added post emulsification, allowing for easy final viscosity control.

Hand Cream

| Phase | Trade Name | INCI Ingredients | Function | % w/w | Supplier |
| --- | --- | --- | --- | --- | --- |
|  | Naissance 210 Grapeseed | Vitis Vinifera (Grape) Seed Oil | Oil/ Moisturising | 2.00 | Naissance |
|  | Myritol 318 | Caprylic/Capric Triglyceride | Emollient | 2.00 | BASF |
| F | Phenoxyethanol RCH | Phenoxyethanol | Preservative | 0.60 | Schulke & Mayr |
| G | Sensiva SC10 | Caprylyl Glycol Ethylhexylglycerin | Preservative | 0.40 | Schulke & Mayr |
|  |  | Parfum | Fragrance | QS |  |
| H | Lancos SC PAN | Panthenol (Vit B5) | Active | 0.50 | OQEMA |
|  | Deionised Water | Aqua | Solvent | 2.00 |  |
| I | Texique HE10 | Sodium Acrylate/Sodium Acryloyldimethyl Taurate Copolymer (and) C13-15 Alkane (and) Coco-Glucoside | Thickener/ Emulsifier | 1.50 | Scott Bader |
| J | Citric Acid Solution | Citric Acid | pH Adjuster | QS |  |

Method

- Weigh out phase A and mix until completely dissolved.

- Whilst homogenising, add phase B and mix until dispersed.

- Add phase C and mix. Start heating to 80°C.

- At temperature, add phase D to water phase and homogenise gently to disperse.

- Weigh out phase E and heat to 80°C with mixing. At 80°C add phase F and mix until uniform.

- Add oil phase to water phase whilst homogenising. Homogenise until smooth and uniform.

- 7. Cool to below 40°C with slow mixing.

- 8. At below 40°C add phase G and mix until uniform.

- 9. Blend phase H together until clear solution and then add to main batch and mix until uniform.

- 10. Add phase I and mix to disperse. Homogenise gently to achieve smooth thick emulsion.

- 11. Adjust to pH 5.50 – 6.50 with citric acid solution (phase J).

The information in this publication is based on laboratory testing and is believed to be accurate and is given in good faith, but no representation or warranty as to its completeness or accuracy is made. Suggestions for uses or applications are only opinions and are not intended for design purposes. Users are responsible for determining the suitability of these products for their own particular purpose and assume all risk and liability whether used singly or in combination with other materials. No representation or warranty, expressed or implied, is made with respect to information or products including, without limitation, warranties of merchantability, fitness for a particular purpose, non-infringement of any third-party patent or other intellectual property rights.